NORD GRAND PROGRAMS - FACTORY PRESETS          AUGUST 31, 2019

COMPILED FOR PERSONAL USE OF PREPARER (NOT AFFILIATED WITH CLAVIA/NORD)

MAY CONTAIN ERRORS AND OMISSIONS

GENERAL NOTES:

(1)  THIS DOCUMENT WAS COMPILED FOR THE PERSONAL USE OF THE PREPARER. HE IS NOT AFFILIATED IN ANY WAY WITH CLAVIA/NORD NOR HAS CLAVIA/NORD REVIEWED OR COMMENTED
       UPON THIS DOCUMENT. IT IS MADE AVAILABLE ON AN “AS IS” BASIS FOR INFORMATION ONLY AND MAY CONTAIN ERRORS AND OMISSIONS.

(2)  THIS DOCUMENT LISTS THE PIANO AND SAMPLE SYNTH SOUNDS, EFFECTS, PARAMETERS AND SETTINGS CONTAINED IN THE FACTORY PRESET PROGRAMS LOADED INTO THE PREPARER’S NORD 
       GRAND WHEN RECEIVED NEW IN AUGUST 2019. SOME OF THOSE ELEMENTS WERE NOT ACTIVE IN CERTAIN PRESET PROGRAMS AND THEREFORE ARE LEFT BLANK IN THE DOCUMENT. 
       (HOWEVER, MOST IF NOT ALL OF THOSE SILENT FEATURES ARE IN FACT ACTUALLY PRELOADED, ALTHOUGH INITIALLY DORMANT AND NOT SHOWN ON THE OLED DISPLAY; NEVERTHELESS, 
       THEY MAY BE MANUALLY ACTIVATED IF DESIRED.) YOU CAN DOWNLOAD A LIST OF YOUR OWN PIANO AND SAMPLE SYNTH SOUNDS, INCLUDING DORMANT ONES, BUT EXCLUDING SETTINGS, 
       BY OPENING NORD SOUND MANAGER, SELECTING “FILE,” THEN CLICKING ON “EXPORT SOUND LISTS.”

(3)  THE NEW “WHITE GRAND” PIANO SOUND WAS NOT INCLUDED IN THIS INSTRUMENT WHEN RECEIVED AND THEREFORE DOES NOT APPEAR IN THE FOLLOWING CHART.

(4)  PIANO AND SAMPLE SYNTH LEVELS ARE GIVEN IN DECIBELS (DB), BETWEEN “OFF” (THE MINIMUM NUMERICAL SETTING [ABOVE “OFF”] THAT DISPLAYS IS -84.2DB) AND 0.0DB (MAXIMUM), 
       CORRESPONDING TO THE “0” AND “10” POSITIONS ON THE KNOB, RESPECTIVELY); KNOB SETTINGS DO NOT CORRESPOND TO DB LEVELS.

(5)  PROGRAM B:51 IS IN MONO (AS INDICATED IN THE PNO COLUMN). ALL OTHERS ARE IN STEREO. NOT TO BE CONFUSED WITH THE PIANO SOUND CALLED “EGRAND 3 MONO SML.”

(6)  ALL PIANO AND SAMPLE SYNTH SOUNDS CONTAIN PEDAL SUSTAIN, EXCEPT THE SAMPLE SYNTH SOUND IN PROGRAM E:45.

(7)  SOFT RELEASE (IN THE PIANO SECTION) IS ACTIVE IN ONLY 14 PROGRAMS AND ITS NAME IS SPELLED OUT IN THE FOLLOWING CHART. IT IS DIFFERENT THAN THE STRING RES (SR) EFFECT.

(8)  KNOB-ADJUSTABLE PARAMETERS (INCLUDING VOLUME LEVELS) OPERATE AT THE INITIAL FACTORY PRESET LEVELS INDICATED, REGARDLESS OF THE PHYSICAL KNOB POSITIONS; MOVING A
       KNOB WILL CAUSE THAT SETTING TO CHANGE INSTANTLY TO THE KNOB’S PHYSICAL POSITION. TO SEE A PARAMETER’S STORED LEVEL WITHOUT CHANGING IT, FIRST MAKE SURE THE 
       PROGRAM IS IN UNEDITED MODE, THEN PRESS AND HOLD THE SHIFT BUTTON AND MOVE THE KNOB. (IF THE DISPLAY SHOWS A SMALL “E” IN A SQUARE NEXT TO THE PROGRAM NUMBER, 
       THE PROGRAM HAS BEEN EDITED; TO RETURN TO THE UNEDITED STORED PROGRAM, MOVE THE PROGRAM KNOB A CLICK TO THE RIGHT OR LEFT, THEN BACK.)

(9)  WHEN A PIANO OR SAMPLE SYNTH SOUND IS REPLACED IN THE INSTRUMENT BY A SUBSTITUTION OR A RELINK OPERATION, ALL PROGRAM SETTINGS ARE CARRIED FORWARD TO THE NEW 
       SOUND.

KEY TO SOUND AND EFFECT SETTINGS (BUTTONS):

SYNT/PNO AND PNO/SYNT = KBD SPLIT AT THE INDICATED NOTE (F2, C3, F3, C4, F4, C5, F5, C6 OR F6). WIDTH OF SPLIT POINT CROSS-FADE IS INDICATED AS 1, 6 OR 12. FOR EXAMPLE, A SPLIT WITH             
       THE SAMPLE SYNTH SOUND AT THE LEFT AND THE PIANO SOUND TO THE RIGHT, POSITIONED AT NOTE F3 AND HAVING A WIDTH OF 6, WOULD BE SHOWN AS SYNT/PNOF3W6. KBD SPLITS ARE
       NOTED IN THE PIANO COLUMN.

KT = KBD TOUCH (LIGHT, MEDIUM OR HEAVY). SHOWN AS KTL, KTM OR KTH.

TS, TM, TB, TD1 AND TD2 = TIMBRE (SOFT, MID, BRIGHT, DYNO1 AND DYNO2).

SR = STRING RES (NOT SOFT RELEASE, A DIFFERENT EFFECT USED IN ONLY 14 PROGRAMS).

PN = PEDAL NOISE.

FLTR = FILTER SETTING IN SAMPLE SYNTH SECTION.

AMP = SETTING IN SAMPLE SYNTH SECTION (HAS NOTHING TO DO WITH AMP/COMP UNIT). MAKES KEYS VELOCITY SENSITIVE.

FX1 AND FX2 = SELECTED SPECIAL EFFECTS, AS LISTED IN PIANO AND SAMPLE SYNTH COLUMNS. NOTE THAT EACH EFFECT IN FX2 MAY ALSO BE SECONDARILY DESIGNATED AS DEEP, AS IN   
       “CHOR2DEEP”.

DELAY (DEL) = ECHO EFFECT. INFORMATION IS SHOWN FOR DRY/WET MIX (D/W), FEEDBACK (FB) LEVEL (OFF, 1,2 OR 3) AND TEMPO. TEMPO IS SHOWN IN BEATS-PER-MINUTE, PRECEDED BY THE LETTER “T.”
EQ = EQUALIZER. INCLUDES, BASS, MID-RANGE, FREQUENCY (MID-RANGE SWEEP CONTROL), AND TREBLE.

SMALL, JC, TWIN, DIST AND COMP = AMPLIFIER SIMULATION, DISTORTION (OVERDRIVE) AND COMPRESSOR SETTINGS IN AMP/COMP (A/C) UNIT. CORRESPONDING DRIVE LEVEL (D) AND COMP
       LEVEL (C) DIAL PARAMETERS ARE SHOWN IN A/C COLUMN).

HALL, STAGE OR ROOM = REVERB TYPE. EACH TYPE OF REVERB MAY ALSO BE SECONDARILY DESIGNATED AS BRIGHT (BRT), AS IN “ROOMBRT.” REVERB IS A GLOBAL SETTING, APPLICABLE TO BOTH
       PIAN0 AND SAMPLE SYNTH WHENEVER BOTH ARE USED, BUT LISTED ONLY ONCE FOR EACH PROGRAM (IN THE PIANO COLUMN, UNLESS NO PIANO SOUND IS USED).

KEY TO PARAMETER SETTINGS (GENERALLY KNOBS, BUT ALSO INCLUDES DELAY TEMPO TAP BUTTON):

PL = PIANO LEVEL, GIVEN IN DB (DOES NOT CORRESPOND TO KNOB SETTING – SEE GENERAL NOTE 4 ABOVE).

SSL = SAMPLE SYNTH LEVEL, GIVEN IN DB (DOES NOT CORRESPOND TO KNOB SETTING – SEE GENERAL NOTE 4 ABOVE).

ATK = ATTACK PARAMETER, GIVEN IN MILLISECONDS, EXCEPT WHEN DESIGNATED IN SECONDS (S).

DC/R = DECAY (D) OR RELEASE (R), GIVEN IN MILLISECONDS, EXCEPT WHEN DESIGNATED IN SECONDS (S).

FX1 AND FX2 = SPECIAL EFFECTS RATES (MOST ARE GIVEN IN HERTZ, WHILE THE OLED DISPLAY SHOWS NO UNIT OF MEASUREMENT FOR 11 OTHERS).

BASS, MID AND TRB = BASS, MID-RANGE AND TREBLE LEVELS IN EQ SECTION, GIVEN IN DB CORRESPONDING TO KNOB SETTINGS.

FRQ = ADJUSTABLE MID-RANGE FREQUENCY IN EQ SECTION, GIVEN IN HERTZ, EXCEPT WHEN DESIGNATED IN KILOHERTZ (kHz).

DELAY PARAMETERS = D/W, FB AND TEMPO. SEE DESCRIPTIONS IN “DELAY” UNDER “KEY TO SOUND AND EFFECT SETTINGS” ABOVE.

DRIVE/COMP PARAMETER (IN AMP/COMP UNIT) = LEVEL OF DRIVE EFFECT (D) OR COMPRESSOR EFFECT (C). DRIVE APPLIES ONLY TO SMALL, JC, TWIN OR DIST (BUT NOT COMP) EFFECTS. “ACTIVE” 
       LAMP ILLUMINATES WHEN ONE OF THESE SETTINGS (OTHER THAN COMP) IS SELECTED AND SETTING IS HIGHER THAN ZERO.

REV = REVERB MIX (DRY/WET); NOTED IN PIANO COLUMN WHEN ACTIVE, UNLESS NO PIANO SOUND IS USED IN A CERTAIN PROGRAM.

| PROGRAM | PIANO | SAMPLE SYNTH | PL | SSL | ATK | DC/R | FX1 | FX2 | BASS | MID | FRQ | TRB | DELAY | A/C | REV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A:11 ROYAL GRAND 3D | ROYAL GRAND 3D XL KTH, SR, PN |  | -2.0 |  |  |  |  |  |  |  |  |  |  |  | N/A |
| A:12 AMBER UPRIGHT | AMBER UPRIGHT XL KTH, SR, PN |  | -2.0 |  |  |  |  |  |  |  |  |  |  |  | N/A |
| A:13 NEFERTITI MK I | EP8 NEFERTITI XL KTM, TM, PN, PAN1, VIBE, STAGE |  | -2.0 |  |  |  | 2.6 | 2.9 |  |  |  |  |  |  | 3.0 |
| A:14 MM CHOIR |  | MEN+WOMEN MM AMP, HALLBRT |  | 7.0 | 0.0 | R210 |  |  |  |  |  |  |  |  | 3.6 |
| A:15 GRAND STRINGS | ROYAL GRAND 3D XL KTM, SR, PN, HALL | SOFT STRINGS LEGATO AMP | -2.0 | -11.6 | 0.0 | R683 |  |  |  |  |  |  |  |  | 4.0 |
| A:21 VELVET GRAND | VELVET GRAND LRG KTM, SR, PN, ROOM |  | -2.0 |  |  |  |  |  |  |  |  |  |  |  | 3.4 |
| A:22 BABY UPRIGHT | BABY UPRIGHT MED KTM, SR, PN, ROOM |  | -2.0 |  |  |  |  |  |  |  |  |  |  |  | 5.4 |
| A:23 DYNO TINES | EP5 BRIGHTTINES XL KTM, TD2, PN, CHOR2DEEP, EQ, DEL, COMP, HALLBRT |  | -2.0 |  |  |  |  | 0.4 | 0.0 | -6.0 | 8.0 kHz | -1.2 | D/W1.2, FB2, T146 | C1.3 | 2.4 |
| A:24 MARIMBA | MARIMBA KTM, SOFT RELEASE, STAGE |  | 0.0 |  |  |  |  |  |  |  |  |  |  |  | 2.0 |
| A:25 BASS GRAND SPLIT | VELVET GRAND LRG SYNT/PNOF3W6, KTH, SR, PN, ROOMBRT | CONTRA BASS PLUCKED AMP, COMP | -2.0 | -2.0 | 0.0 | R33 |  |  |  |  |  |  |  | C3.3 | 4.0 |
| A:31 BRIGHT GRAND | BRIGHT GRAND MED KTM, SR, PN, STAGEBRT |  | -2.0 |  |  |  |  |  |  |  |  |  |  |  | 2.7 |
| A:32 BAMBINO UPRIGHT | BAMBINO UPRIGHT LRG KTM, SR, PN, STAGE |  | -2.0 |  |  |  |  |  |  |  |  |  |  |  | 3.0 |
| A:33 AMPED MKV | EP4 MK5 80s LRG KTM, TM, VIBE, EQ, TWIN, STAGEBRT |  | -2.0 |  |  |  |  | 1.6 | +6.0 | 0.0 | 999 | +7.0 |  | D1.4 | 2.3 |
| A:34 CHAMBER STRINGS |  | CHAMBERSTR LEG HIVIB AMP, STAGE |  | -6.2 | 0.0 | R120 |  |  |  |  |  |  |  |  | 4.3 |
| A:35 GRAND PAD | ROYAL GRAND 3D XL KTM, SR, PN, COMP, HALL | WARMPAD1 AMP, EQ | -2.0 | 0.0 | 0.0 | R368 |  |  | -2.5 | -7.8 | 520 | 0.0 |  | C5.0 | 3.9 |

| PROGRAM | PIANO | SAMPLE SYNTH | PL | SSL | ATK | DC/R | FX1 | FX2 | BASS | MID | FRQ | TRB | DELAY | A/C | REV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A:41 GRAND IMPERIAL | GRAND IMPERIAL LRG KTH, SR, PN, HALLBRT |  | -1.0 |  |  |  |  |  |  |  |  |  |  |  | 2.1 |
| A:42 MELLOW UPRIGHT | MELLOW UPRIGHT MED KTM, SR, PN, STAGE |  | -2.0 |  |  |  |  |  |  |  |  |  |  |  | 2.5 |
| A:43 FRONTIERS MK I | EP2 IDEAL TIMBRE LRG KTM, TB, PN, TREM1, PHAS1, EQ, STAGEBRT |  | -2.0 |  |  |  | 4.3 | 3.6 | -8.5 | -5.5 | 758 | +3.2 |  |  | 3.1 |
| A:44 BIG POLYSYNTH |  | HORNSAW FLTR, EQ, HALLBRT |  | -2.0 | 61 | R1.13S |  |  | 0.0 | -4.0 | 306 | 0.0 |  |  | 4.5 |
| A:45 BELL GRAND | SILVER GRAND LRG KTH, ROOM | METALLOPHONE FLTR, AMP | 0.0 | -11.4 | 0.0 | R173 |  |  |  |  |  |  |  |  | 5.3 |
| A:51 SILVER GRAND | SILVER GRAND LRG KTH, SR, PN, STAGE |  | -2.0 |  |  |  |  |  |  |  |  |  |  |  | 2.9 |
| A:52 WAH CLAV | CLAVINET D6 DA KTL, TM, WAH, ROOM |  | -1.0 |  |  |  | 2.4 |  |  |  |  |  |  |  | 1.5 |
| A:53 WURLITZER | WURLITZER 2 XL KTM, TREM1, EQ, ROOM |  | -2.0 |  |  |  | 4.4 |  | -2.0 | +4.0 | 2.0 kHz | 0.0 |  |  | 4.0 |
| A:54 MELLOTRON FLUTES |  | FLUTE MELLOTRON AMP, STAGE |  | -6.0 | 0.0 | R75 |  |  |  |  |  |  |  |  | 1.8 |
| A:55 SHIMMER UPRIGHT | AMBER UPRIGHT XL KTH, SR, PN, HALLBRT | GRANDMAS UPRIGHT AMP, PAN2, CHOR1DEEP, EQ, DEL | -2.0 | -2.7 | 1.49S | R2.36S | 2.1 | 0.7 | -15.0 | 0.0 | 999 | 0.0 | D/W10.0, FB2, T146 |  | 5.0 |
| B:11 STUDIO GRAND 2 | STUDIO GRAND 2 LRG KTM, SR, PN, STAGE |  | -2.0 |  |  |  |  |  |  |  |  |  |  |  | 3.0 |
| B:12 ROMANTIC UPRIGHT | ROMANTIC UPRIGHT MED KTH, SR, PN, HALL |  | -1.0 |  |  |  |  |  |  |  |  |  |  |  | 2.5 |
| B:13 BALLAD EP 1 | BALLAD EP1 SML KTM, STAGEBRT |  | -2.2 |  |  |  |  |  |  |  |  |  |  |  | 2.4 |
| B:14 ACOUSTIC GUITAR |  | AC GUITAR FINGER FLTR, AMP, STAGE |  | -2.0 | 0.0 | R75 |  |  |  |  |  |  |  |  | 4.0 |
| B:15 SLICK BASS SPLIT | BRIGHT GRAND MED SYNT/PNOF3W6, KTH, TS, EQ, DEL, COMP, HALLBRT | CONTRA BASS FING VIB FLTR, CHOR1 | 0.0 | -3.0 | 0.0 | R33 |  | 0.4 | 0.0 | +10.8 | 8.0 kHz | 0.0 | D/W0.9, FB2, T86 | C4.7 | 2.2 |

| PROGRAM | PIANO | SAMPLE SYNTH | PL | SSL | ATK | DC/R | FX1 | FX2 | BASS | MID | FRQ | TRB | DELAY | A/C | REV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B:21 ELECTRIC GRAND | EGRAND 3 MONO SML KTM, STAGEBRT |  | -1.0 |  |  |  |  |  |  |  |  |  |  |  | 2.5 |
| B:22 HONKY TONK | HONKYTONKUPRIGHT MED KTM, SR, PN, ROOMBRT |  | 0.0 |  |  |  |  |  |  |  |  |  |  |  | 4.0 |
| B:23 FM TINES | DX7 FULLTINES LRG KTM, CHOR1, STAGEBRT |  | -4.0 |  |  |  |  | 0.6 |  |  |  |  |  |  | 3.2 |
| B:24 POP BRASS |  | BRASS SECTION POP 3 FLTR, AMP, EQ, STAGEBRT |  | -5.0 | 0.0 | R50 |  |  | +1.0 | -2.0 | 685 | 0.0 |  |  | 2.1 |
| B:25 EP BASS SPLIT | EP8 NEFERTITI XL SYNT/PNOF3W12, KTH, TD1, PN, PAN1, CHOR2, EQ, STAGE | FINGER BASS ALEM FLTR, AMP, COMP | -2.0 | -5.0 | 0.0 | R33 | 2.7 | 0.4 | 0.0 | -2.2 | 8.0 kHz | 0.0 |  | C4.1 | 2.5 |
| B:31 FORTEPIANO | MOZART FORTEPNO MED KTM, SR, PN, STAGE |  | -2.0 |  |  |  |  |  |  |  |  |  |  |  | 2.5 |
| B:32 BLACK UPRIGHT | BLACK UPRIGHT MED KTM, SR, PN, ROOM |  | -1.6 |  |  |  |  |  |  |  |  |  |  |  | 4.0 |
| B:33 LOGICAL WURLY | WURLITZER 2 XL KTM, TM, CHOR1DEEP, EQ, STAGE |  | -3.0 |  |  |  |  | 1.6 | +6.5 | -3.5 | 482 | +3.0 |  |  | 1.6 |
| B:34 STRING QUARTET |  | STRING QUARTET LEG VIB AMP, STAGE |  | -4.0 | 0.0 | R75 |  |  |  |  |  |  |  |  | 3.5 |
| B:35 DUO LINES | ROYAL GRAND 3D XL KTM, SR, PN, STAGE | FINGER BASS P 58 FLTR, AMP | -2.0 | -6.0 | 0.0 | R33 |  |  |  |  |  |  |  |  | 2.7 |
| B:41 SOFT ROYAL | ROYAL GRAND 3D XL KTH, TS, SR, PN, EQ, ROOM |  | 0.0 |  |  |  |  |  | -2.0 | +12.0 | 8.0 kHz | +3.0 |  |  | 4.9 |
| B:42 GRAND UPRIGHT | GRAND UPRIGHT LRG KTM, SR, PN |  | 0.0 |  |  |  |  |  |  |  |  |  |  |  | N/A |
| B:43 SILENCE MK I | EP5 BRIGHTTINES XL KTM, TS, PN, VIBE, STAGE |  | -3.0 |  |  |  |  | 3.1 |  |  |  |  |  |  | 2.8 |
| B:44 CLASSICAL GUITAR |  | CLASSICAL GUITAR FLTR, AMP, HALL |  | 0.0 | 0.0 | R75 |  |  |  |  |  |  |  |  | 4.5 |
| B:45 ROYAL VOICES | ROYAL GRAND 3D XL KTH, SR, PN, HALLBRT | MEN+BOYS 00 SOFT AMP | -2.0 | -19.6 | 0.0 | R432 |  |  |  |  |  |  |  |  | 4.6 |

| PROGRAM | PIANO | SAMPLE SYNTH | PL | SSL | ATK | DC/R | FX1 | FX2 | BASS | MID | FRQ | TRB | DELAY | A/C | REV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B:51 FORGOTTEN 78 | VELVET GRAND LRG MONO, KTM, TM, SR, PN, VIBEDEEP, EQ, DEL, SMALL, STAGE |  | -4.0 |  |  |  |  | 3.1 | -15.0 | +15.0 | 1.7 kHz | -9.2 | D/W3.5, FBOFF, T480 | D0.0 | 2.9 |
| B:52 CLAV ROOM | CLAVINET D6 CB KTL, TD1, EQ, DEL, TWIN, ROOMBRT |  | -3.0 |  |  |  |  |  | +4.0 | +2.0 | 5.0 kHz | +3.0 | D/W2.6, FBOFF, T180 | D0.3 | 7.0 |
| B:53 DIGIGRAND CHORUS | DIGIGRAND 2 SML KTL, SOFT RELEASE, CHOR1DEEP, STAGEBRT |  | 0.0 |  |  |  |  | 0.6 |  |  |  |  |  |  | 2.8 |
| B:54 3 VLNS CHAMBERLIN |  | 3 VIOLINS CHAMBERLIN AMP, EQ, STAGE |  | -3.5 | 0.0 | R75 |  |  | +1.0 | -5.0 | 1.5 kHz | 0.0 |  |  | 2.0 |
| B:55 SILVER FLUTES | SILVER EP SML KTM, STAGEBRT | FLUTE MELLOTRON AMP | -3.0 | -8.0 | 0.0 | R75 |  |  |  |  |  |  |  |  | 3.0 |
| C:11 SILVER CHORUS | SILVER GRAND LRG KTM, TB, SR, PN, CHOR1, HALLBRT |  | -1.4 |  |  |  |  | 0.3 |  |  |  |  |  |  | 2.8 |
| C:12 HARPSICHORD | FRENCH HARPSI 1D KTM, STAGE |  | 0.0 |  |  |  |  |  |  |  |  |  |  |  | 5.0 |
| C:13 SPAIN MK I | EP6 SPARKLETOP XL KTM, PN, VIBE, EQ, TWIN, STAGE |  | -1.6 |  |  |  |  | 1.7 | -8.0 | +7.0 | 881 | -9.5 |  | D0.0 | 2.8 |
| C:14 RICH SYNTH BRASS |  | RICH SYNTH BRASS FLTR, AMP, STAGE |  | -2.0 | 0.0 | R75 |  |  |  |  |  |  |  |  | 3.1 |
| C:15 BRANDENBURG | FRENCH HARPSI 1D KTM, SR, STAGE | STUDIOSTRINGS MARC VIB AMP, EQ | -4.0 | -8.2 | 0.0 | R120 |  |  | 0.0 | 0.0 | 881 | +6.0 |  |  | 7.5 |
| C:21 HOOKED UP CP | EGRAND 3 MONO SML KTM, TB, FLANG, DIST, STAGEBRT |  | -4.0 |  |  |  |  | 2.7 |  |  |  |  |  | D0.8 | 3.8 |
| C:22 ROCK UPRIGHT | AMBER UPRIGHT XL KTL, TM, VIBE, EQ, COMP, ROOM |  | -3.0 |  |  |  |  | 1.5 | -5.0 | -1.0 | 2.9 kHz | 0.0 |  | C1.5 | 5.4 |
| C:23 BUTTERFLY MK I | EP1 DEEP TIMBRE LRG KTM, TS, PN, PAN2, STAGE |  | -2.5 |  |  |  | 3.3 |  |  |  |  |  |  |  | 2.1 |

| PROGRAM | PIANO | SAMPLE SYNTH | PL | SSL | ATK | DC/R | FX1 | FX2 | BASS | MID | FRQ | TRB | DELAY | A/C | REV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C:24 FULL ORCHESTRA |  | WIND-STRING-BRASS FLTR, AMP, HALL |  | -6.0 | 0.0 | R368 |  |  |  |  |  |  |  |  | 5.0 |
| C:25 GOSPEL PAD | HYBRID EP SML KTM, SOFT RELEASE, PAN1, EQ, HALLBRT | WIDEPAD PPGWv FLTR, CHOR2 | -1.6 | -8.5 | 0.0 | R432 | 2.1 | 2.7 | 0.0 | -2.2 | 589 | +3.0 |  |  | 3.1 |
| C:31 KEITH IN KOLN | GRAND IMPERIAL LRG KTH, SR, PN, EQ, STAGEBRT |  | -1.4 |  |  |  |  |  | -2.0 | +7.2 | 5.0 kHz | 0.0 |  |  | 6.0 |
| C:32 SALOON UPRIGHT | SALOON UPRIGHT SML KTM, PN, EQ, ROOMBRT |  | -1.0 |  |  |  |  |  | 0.0 | -4.0 | 375 | 0.0 |  |  | 1.7 |
| C:33 VINTAGE WURLY | WURLITZER 2 XL KTM, SOFT RELEASE, EQ, DEL, TWIN, ROOMBRT |  | -3.0 |  |  |  |  |  | -15.0 | -3.8 | 200 | 0.0 | D/W3.0 FB1, T160 | D3.5 | 3.7 |
| C:34 JAZZ GUITAR |  | JAZZ GUITAR FLT, AMP, EQ, DEL, TWIN, STAGE |  | 0.0 | 0.0 | R33 |  |  | +8.0 | +1.5 | 1.8 kHz | -8.0 | D/W1.2, FB1, T88 | D1.0 | 5.0 |
| C:35 CELESTIAL MELLOW | MELLOW UPRIGHT MED KTM, SR, PN, EQ, DEL, COMP, HALLBRT | CELESTIAL PAD FLTR | -2.0 | -14.5 | 0.0 | R792 |  |  | -2.0 | -2.0 | 447 | +4.0 | D.W2.3, FB1, T82 | C2.5 | 4.5 |
| C:41 1973 GRAND | ROYAL GRAND 3D XL KTM, TM, SR, PN, EQ, COMP, ROOMBRT |  | -4.0 |  |  |  |  |  | -5.0 | +9.0 | 6.6 kHz | +4.0 |  | C2.8 | 4.5 |
| C:42 PRODUCER UPRIGHT | BAMBINO UPRIGHT LRG KTL, TB, SR, PN, EQ, DEL, COMP, STAGE |  | -2.0 |  |  |  |  |  | +1.0 | -11.0 | 604 | +2.2 | D/W2.0, FB1, T120 | C8.0 | 2.7 |
| C:43 OVERDRIVE MK I | EP2 IDEAL TIMBRE LRG KTM, TM, PN, VIBE, EQ, DIST, STAGEBRT |  | -10.6 |  |  |  |  | 2.7 | -0.5 | +9.8 | 1.6 kHz | +4.2 |  | D2.0 | 1.9 |
| C:44 CLARINET SECTION |  | 3-CLARINETS LEGATO AMP, HALL |  | -2.0 | 0.0 | R75 |  |  |  |  |  |  |  |  | 5.0 |
| C:45 AL ANDALUS | SILVER GRAND LRG KTM, SOFT RELEASE, SR, PN, DEL, HALL | CLASSICAL GUITAR FLTR, AMP, EQ | -6.0 | -0.6 | 0.0 | R75 |  |  | 0.0 | -3.0 | 356 | 0.0 | D/W0.4, FB2, T154 |  | 5.2 |

| PROGRAM | PIANO | SAMPLE SYNTH | PL | SSL | ATK | DC/R | FX1 | FX2 | BASS | MID | FRQ | TRB | DELAY | A/C | REV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C:51 VELVET BOP | VELVET GRAND LRG KTM, TM, SR, PN, EQ, COMP, ROOM |  | -1.0 |  |  |  |  |  | -5.0 | +2.0 | 1.2 kHz | -5.0 |  | C2.5 | 2.7 |
| C:52 WAITS UPRIGHT | BLACK UPRIGHT MED KTM, TM, SR, PN, EQ, COMP, STAGEBRT |  | -1.4 |  |  |  |  |  | 0.0 | -7.0 | 365 | +2.0 |  | C4.0 | 2.4 |
| C:53 DYNO MK II | EP3 SHALLOW CLOSE LRG KTM, TD2, PN, CHOR2DEEP, HALLBRT |  | -3.0 |  |  |  |  | 0.4 |  |  |  |  |  |  | 3.4 |
| C:54 CATHEDRAL ORGAN |  | CATHEDRAL ORGAN 2 HALL |  | -8.0 | 0.0 | R75 |  |  |  |  |  |  |  |  | 5.8 |
| C:55 BABY BELL | BABY UPRIGHT MED KTH, SR, PN, HALLBRT | GLOCKENSPIEL FLTR, AMP, EQ, DEL | -2.0 | -14.5 | 0.0 | R432 |  |  | 0.0 | -2.0 | 7.0 kHz | -1.8 | D/W0.6, FB1, T146 |  | 2.0 |
| D:11 ESBJORN GRAND | ROYAL GRAND 3D XL KTM, TB, SR, PN, CHOR1, EQ, COMP, HALL |  | -1.0 |  |  |  |  | 0.1 | -8.8 | +5.5 | 3.3 kHz | -1.8 |  | C3.3 | 2.7 |
| D:12 FULL CLAV | CLAVINET D6 CA KTM, TD1D2, STAGE |  | 0.0 |  |  |  |  |  |  |  |  |  |  |  | 3.0 |
| D:13 OVERDRIVE WURLY | WURLITZER 1 KTL, TB, EQ, DIST, ROOM |  | -5.0 |  |  |  |  |  | 0.0 | -6.0 | 1.4 kHz | -3.0 |  | D2.4 | 2.0 |
| D:14 PICKED ACOUSTIC |  | AC GUITAR PICK FLTR, AMP, STAGE |  | -1.0 | 0.0 | R75 |  |  |  |  |  |  |  |  | 4.0 |
| D:15 60s PNO STRINGS | ROMANTIC UPRIGHT MED KTM, TB, SR, PN, COMP, ROOMBRT | STUDIOSTRINGS LEG VIB AMP, EQ, DEL | -1.0 | -16.2 | 0.0 | R244 |  |  | -8.8 | +10.0 | 3.0 kHz | -5.0 | D/W4.0, FB1, T160 | C4.0 | 8.0 |
| D21 ENHANCED SILVER | SILVER GRAND LRG KTH, TB, SR, PN, EQ, COMP, ROOMBRT |  | 0.0 |  |  |  |  |  | -7.2 | -6.0 | 2.5 kHz | 0.0 |  | C5.7 | 4.6 |
| D:22 FELT UPRIGHT | MELLOW UPRIGHT MED KTL, TS, SR, PN, EQ, COMP, STAGE |  | -2.0 |  |  |  |  |  | 0.0 | -2.0 | 1.6 kHz | 0.0 |  | C5.0 | 3.6 |

| PROGRAM | PIANO | SAMPLE SYNTH | PL | SSL | ATK | DC/R | FX1 | FX2 | BASS | MID | FRQ | TRB | DELAY | A/C | REV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D:23 SMOOTH MK I | EP8 NEFERTITI XL KTM, CHOR2, COMP, STAGE |  | 0.0 |  |  |  |  | 0.6 | 0.0 | 0.0 | 999 | 0.0 |  | C3.8 | 2.4 |
| D:24 DYNAMIC FILTER |  | A1 THE PROPHET FLTR, STAGE |  | -8.0 | 0.0 | R432 |  |  |  |  |  |  |  |  | 2.3 |
| D:25 SMOOTHEST BALLAD | BALLAD EP1 SML KTH, TS, CHOR1, DEL, HALLBRT | SMALL CHOIR MALE OH SOFT FLTR, EQ | -3.0 | -15.2 | 0.0 | R792 |  | 0.5 | 0.0 | 0.0 | 2.9 kHz | +4.2 | D/W1.5, FB1, T146 |  | 2.9 |
| D:31 BRIGHT CHORUS CP | EGRAND 3 MONO SML KTM, TM, CHOR1, EQ, STAGEBRT |  | -4.7 |  |  |  |  | 1.0 | 0.0 | +7.0 | 6.1 kHz | 0.0 |  |  | 5.0 |
| D:32 PERSIAN UPRIGHT | BAMBINO UPRIGHT LRG KTM, TB, ROOMBRT | PERSIAN SANTUR FLTR, AMP | -1.0 | -19.6 | 0.0 | R173 |  |  |  |  |  |  |  |  | 7.1 |
| D:33 VIBEY MK II | EP3 SHALLOW CLOSE LRG KTM, TM, VIBE, EQ, TWIN, STAGE |  | 0.0 |  |  |  |  | 4.3 | +15.0 | 0.0 | 494 | 0.0 |  | D0.6 | 2.5 |
| D:34 CHORUS CASTER |  | E GUITAR S 62 FLTR, AMP, CHOR2DEEP, EQ, DEL, COMP, STAGE |  | -1.4 | 0.0 | R50 |  | 0.7 | +2.0 | -2.8 | 604 | 0.0 | D/W1.2, FB1, T146 | C5.0 | 3.8 |
| D:35 GRAND STRING PAD | GRAND ROYAL 3D XL KTM, TB, SR, PN, DEL, HALLBRT | STUDIOSTRINGS LEG VIB FLTR, AMP, PAN1, EQ | -2.0 | -8.0 | 1.38S | R1.30S | 0.1 |  | -6.0 | -5.5 | 507 | +0.8 | D/W1.3, FB1, T96 |  | 5.0 |
| D:41 TINY GRAND | STUDIO GRAND 2 LRG KTM, SR, PN, EQ, DEL, COMP, ROOM |  | -2.5 |  |  |  |  |  | -1.5 | +8.2 | 6.1 kHz | +2.0 | D/W2.5, FBOFF, T300 | C3.1 | 3.2 |
| D:42 PIANO FOR KIDS | BLACK UPRIGHT MED KTM, TB, SR, ROOM | TOY PIANO FLTR, AMP | -2.0 | -13.0 | 0.0 | R173 |  |  |  |  |  |  |  |  | 4.4 |
| D:43 SOFT MKI | EP1 DEEP TIMBRE LRG KTM, TS, PN, TREM1, DIST, HALL |  | -4.7 |  |  |  | 4.3 |  |  |  |  |  |  | D0.5 | 2.0 |
| D:44 CELESTE MAGIC |  | CELESTE FLTR, AMP, EQ, DEL, HALL |  | -1.0 | 0.0 | R173 |  |  | -15.0 | -15.0 | 200 | -15.0 | D/W2.7, FB1, T96 |  | 10.0 |
| D:45: RUBBA SILK PAD | DX RUBBATINES LRG KTM, TB, HALL | SILKWORM JX-3P FLTR, AMP, CHOR2, EQ | -2.0 | -8.0 | 456 | R1.70S |  | 0.8 | -15.0 | +6.0 | 8.0 kHz | -15.0 |  |  | 5.3 |

| PROGRAM | PIANO | SAMPLE SYNTH | PL | SSL | ATK | DC/R | FX1 | FX2 | BASS | MID | FRQ | TRB | DELAY |  | A/C |  | RV |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D:51 UP CLOSE GRAND | ROYAL GRAND 3D XL KTM, TB, SR, PN, EQ |  | 0.0 |  |  |  |  |  | -7.8 | +8.2 | 200 | +4.0 |  |  |  |  |  | N/A |  |
| D:52 BAMBINO PAD | BAMBINO UPRIGHT LRG KTM, TS, SR, PN, HALLBRT | CRISP ATTACK PAD FLTR | -1.0 | -16.5 | 0.0 | R432 |  |  |  |  |  |  |  |  |  |  |  | 2.2 |  |
| D:53 HYBRID EP | HYBRID EP SML KTM, PAN1, CHOR1, STAGEBRT |  | -1.0 |  |  |  | 2.1 | 0.7 |  |  |  |  |  |  |  |  |  | 4.0 |  |
| D:54 SOLO CELLO |  | SOLO CELLO 2 VIBRATO AMP, ROOM |  | -4.0 | 0.0 | R33 |  |  |  |  |  |  |  |  |  |  |  | 10.0 |  |
| D:55 BRIGHT PAD | DIGIGRAND 1 SML KTL, SOFT RELEASE, EQ, STAGEBRT | P5 SAGA SYNTH FLTR, AMP | -2.5 | -9.9 | 0.0 | D9.54S |  |  | -5.2 | +12.0 | 1.9 kHz | -8.0 |  |  |  |  |  | 3.7 |  |
| E:11 1955 GRAND | VELVET GRAND LRG KTH, SR, PN, EQ, SMALL, STAGE |  | 0.0 |  |  |  |  |  | -15.0 | -3.2 | 2.7 kHz | -5.0 |  |  |  | D0.0 |  | 2.7 |  |
| E:12 SOUL UPRIGHT | AMBER UPRIGHT XL KTM, SR, PN, VIBEDEEP, EQ, DEL, ROOMBRT |  | -3.0 |  |  |  |  | 2.1 | -6.2 | +10.5 | 8.0 kHz | 0.0 |  | D/W3.0, FBOFF, T320 |  |  |  | 5.9 |  |
| E:13 PEOPLE MK I | EP6 SPARKLETOP XL KTM, TS, PAN2, STAGE |  | -2.0 |  |  |  | 2.6 |  |  |  |  |  |  |  |  |  |  | 2.7 |  |
| E:14 MARCATO STRINGS |  | STUDIOSTRINGS MARC VIB AMP, ROOMBRT |  | -5.0 | 0.0 | R120 |  |  |  |  |  |  |  |  |  |  |  | 6.0 |  |
| E:15 SWEET AND WARM | BALLAD EP1 SML KTH, TS | WARMPAD1 FLTR | -2.5 | -6.4 | 384 | R505 |  |  |  |  |  |  |  |  |  |  |  | N/A |  |
| E:21 CONCERT SILVER | SILVER GRAND LRG KTH, SR, PN, EQ, HALLBRT |  | 0.0 |  |  |  |  |  | -14.0 | +4.5 | 384 | 0.0 |  |  |  |  |  | 3.2 |  |
| E:22 RISING UPRIGHT | AMBER UPRIGHT XL KTL, SOFT RELEASE, DEL, COMP, HALLBRT | GRANDMAS UPRIGHT AMP, EQ | -2.0 | -4.7 | 496 | D3.0 |  |  | -3.5 | -4.0 | 284 | 0.0 |  | D/W10.0, FBOFF, T121 |  | C4.0 |  | 2.2 |  |
| E:23 RUBBA SYN BASS | DX RUBBATINES LRG SYNTPNOC3W12, KTM, COMP, HALLBRT | SYNTHBASS 5 FLTR, CHOR2DEEP | -2.5 | -11.9 | 5.5 | R144 |  | 1.0 |  |  |  |  |  |  |  | C3.0 |  | 2.4 |  |

| PROGRAM | PIANO | SAMPLE SYNTH | PL | SSL | ATK | DC/R | FX1 | FX2 | BASS | MID | FRQ | TRB | DELAY | A/C | REV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E:24 CATHEDRAL ORG 3 |  | CATHEDRAL ORGAN 3 HALL |  | -8.0 | 0.0 | R75 |  |  |  |  |  |  |  |  | 5.1 |
| E:25 HAUT BOIS | FRENCH HARPSI 1D KTM, SR, EQ, STAGE | 3-OBOES LEGATO FLTR, AMP | -5.0 | -5.0 | 0.8 | R75 |  |  | +1.0 | -5.0 | 2.1 kHz | 0.0 |  |  | 4.1 |
| E:31 POLISHED SILVER | SILVER GRAND LRG KTL, TB, SR, VIBE, EQ, |  | 0.0 |  |  |  |  | 1.9 | 0.0 | -5.5 | 1.5 kHz | -2.0 |  |  | N/A |
| E:32 AMBERSPHERE | AMBER UPRIGHT XL KTM, TB, SR, PN, EQ, DEL, HALLBRT | SOFT STRINGS LEG LOVIB FLTR, PAN1 | 0.0 | -25.7 | 6.47S | R1.30S | 0.0 |  | -3.0 | -2.0 | 2.9 kHz | -1.0 | D/W1.1, FB2, T96 |  | 6.0 |
| E:33 90s PIANOS | DIGIGRAND 1 SML KTL, SOFT RELEASE, COMP, STAGEBRT | PIANO KORG M1 FLTR, AMP, DEL | -3.0 | -15.2 | 0.0 | R120 |  |  |  |  |  |  | D/W0.6, FB1, T136 | C4.8 | 3.2 |
| E:34 THEME SYNTH |  | SYNTH BRASS LAYER FLTR, AMP, VIBE, HALLBRT |  | -1.3 | 224 | R1.94S |  | 3.9 |  |  |  |  |  |  | 5.5 |
| E:35 IMPERIAL BRASS | GRAND IMPERIAL LARG KTM, TM, SR, HALL | FRENCH HORNS 1 LEG FLTR, EQ | -2.0 | -4.0 | 0.8 | R173 |  |  | 0.0 | 0.0 | 999 | -5.8 |  |  | 3.5 |
| E:41 SOUNDCHECK GRAND | ROYAL GRAND 3D XL KTM, TS, SOFT RELEASE, SR, PN, EQ, DEL, STAGEBRT |  | 0.0 |  |  |  |  |  | -15.0 | 0.0 | 999 | 0.0 | D/W6.7, FBOFF, T360 |  | 8.0 |
| E:42 AMBER SONATA | AMBER UPRIGHT XL KTM, ROOM | SOLO VIOLIN 2 VIBRATO FLTR, AMP | -2.0 | -9.1 | 246 | R144 |  |  |  |  |  |  |  |  | 10.0 |
| E:43 GRACEFUL DUO | EP1 DEEP TIMBRE LRG KTM, TB, PN, PAN2, VIBE, DIST, STAGEBRT | CLASSICAL GUITAR FLTR, AMP, DEL | -4.0 | -4.0 | 0.0 | R75 | 2.5 | 1.8 |  |  |  |  | D/W0.9, FB1, T106 | D0.7 | 4.6 |
| E:44 GLOCKENSPIEL |  | GLOCKENSPIEL FLTR, AMP, STAGE |  | -6.0 | 0.0 | R173 |  |  |  |  |  |  |  |  | 2.1 |
| E:45 CONTINUO | ITAL HARPSICH 1B STR KTM, SR, HALL | ORGAN FLUTE 3 NO PEDAL SUSTAIN, EQ | -6.6 | -7.0 | 0.0 | R21 |  |  | -0.8 | -4.0 | 250 | 0.0 |  |  | 4.5 |
| E:51 COMPRESSED POP | ROYAL GRAND 3D XL KTL, TB, EQ, COMP, STAGE |  | -6.0 |  |  |  |  |  | -10.8 | -4.2 | 215 | 0.0 |  | C10.0 | 2.0 |
| E:52 AMBER BELLS | AMBER UPRIGHT XL KTM, SOFT RELEASE, SR, PN, HALL | BELLTOWER GEFLE FLTR, AMP | -1.0 | -22.9 | 0.0 | R1.13S |  |  |  |  |  |  |  |  | 4.3 |

| PROGRAM | PIANO | SAMPLE SYNTH | PL | SSL | ATK | DC/R | FX1 | FX2 | BASS | MID | FRQ | TRB | DELAY | A/C | REV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E:53 FUSION MK I | EP1 DEEP TIMBRE LRG KTM, TM, PN, PHAS1, EQ, SMALL, ROOMBRT |  | -2.0 |  |  |  |  | 2.5 | +3.0 | 0.0 | 1.1 kHz | +3.0 |  | D1.6 | 4.2 |
| E:54 CELESTIAL PAD |  | CELESTIAL PAD FLTR, AMP, HALLBRT |  | -5.0 | 13 | R792 |  |  |  |  |  |  |  |  | 5.9 |
| E:55 LODGE EP | EP2 IDEAL TIMBRE LRG SYNT/PNOC3W6, KTM, TB, PN, PAN2, HALL | ROCK GUITAR MELLOTRON AMP, VIBE, EQ, COMP | 0.0 | -3.0 | 0.5 | R75 | 2.7 | 3.3 | +5.0 | -7.0 | 3.3 kHz | -6.5 |  | C4.3 | 2.6 |
| F:11 JOHANSSONS GRAND | STUDIO GRAND 2 LRG KTH, TM, SR, PN, PAN1, EQ, STAGEBRT |  | -2.0 |  |  |  | 0.0 |  | -15.0 | +11.5 | 3.9 kHz | -7.0 |  |  | 4.1 |
| F:12 BEAUTIFUL LOVE | AMBER UPRIGHT XL KTM, TM, SR, VIBEDEEP, SMALL, STAGEBRT | SOLO VIOLIN 2 VIBRATO AMP, EQ | -2.5 | -8.7 | 0.0 | R206 |  | 2.5 | -15.0 | +15.0 | 1.1 kHz | -12.8 |  | D0.8 | 4.4 |
| F:13 RAYS REEDS | WURLITZER 1 KTM, TM, SMALL, ROOM |  | -1.0 |  |  |  |  |  |  |  |  |  |  | D1.0 | 2.4 |
| F:14 FUNKY SYNTH |  | SYNTH TRUMPET MM FLTR, WAH, VIBE [PNO VIBE IS SELECTED BUT PNO IS NOT ACTIVE - ?], ROOMBRT |  | -4.0 | 0.0 | D736 | 1.6 | 1.0 |  |  |  |  |  |  | 4.0 |
| F:15 STACCATO UPRIGHT | ROMANTIC UPRIGHT MED KTL, TB, SR, PN, COMP, HALL | 16-VIOLIN STACCATO AMP, EQ | -5.4 | -4.9 | 0.0 | R1.30S |  |  | +1.2 | -4.8 | 950 | 0.0 |  | C5.0 | 2.0 |
| F:21 DIGITIZED BRIGHT | BRIGHT GRAND MED KTL, TB, SOFT RELEASE, CHOR1, EQ, STAGEBRT |  | -6.0 |  |  |  |  | 0.2 | -7.0 | +6.0 | 8.0 kHz | +2.0 |  |  | 3.2 |
| F:22 COMPRESSED FLUTE | MELLOW UPRIGHT MED KTM, TB, CHOR1, COMP | 3-FLUTES LEGATO FLTR | -6.0 | -6.0 | 1.02S | R75 |  | 0.7 | 0.0 | 0.0 | 999 | 0.0 |  | C6.5 | N/A |
| F:23 DYNO TALES | EP8 NEFERTITI XL KTM, TD2, PN, CHOR1DEEP, HALLBRT | DX7IIC FINETINES FLTR, AMP, PAN2, DEL | -2.0 | -11.6 | 0.0 | R75 | 2.2 | 1.6 |  |  |  |  | D/W10.0, FB1, T120 |  | 4.0 |
| F:24 VIBRAPHONE |  | VIBES FLTR, AMP, TREM1, STAGE |  | -9.1 | 0.0 | R173 | 4.3 |  |  |  |  |  |  |  | 2.4 |

| PROGRAM | PIANO |  | SAMPLE SYNTH |  | PL |  | SSL |  | ATK |  | DC/R |  | FX1 |  | FX2 |  | BASS |  | MID |  | FRQ |  |  | TRB |  |  | DELAY |  |  | A/C |  |  | REV |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F:25 SILVER PAD | SILVER EP SML KTM, TB, CHOR2DEEP, EQ, COMP, STAGE |  | CRISP ATTACK PAD FLTR |  | -3.0 |  | -1.0 |  | 0.0 |  | R206 |  |  |  | 0.3 |  | -8.0 |  | 0.0 |  |  | 482 |  |  | 0.0 |  |  |  |  |  | C4.9 |  |  | 1.7 |  |  |
| F:31 BRIGHT AND SOFT | BRIGHT GRAND MED KTH, TS, SR, PN, EQ, HALLBRT |  |  |  | -1.0 |  |  |  |  |  |  |  |  |  |  |  | 0.0 |  | +8.0 |  | 6.6 kHz |  |  | 0.0 |  |  |  |  |  |  |  |  | 3.0 |  |  |
| F:32 FADING DREAM | MELLOW UPRIGHT MED KTM, TB, SR, PAN1, VIBEDEEP, DEL, HALLBRT |  | WARMPAD1 FLTR |  | 0.0 |  | -25.7 |  | 139 |  | R432 |  | 1.3 |  | 2.6 |  |  |  |  |  |  |  |  |  |  |  | D/W3.5, FB1, T146 |  |  |  |  |  | 2.8 |  |  |
| F:33 THICK TINE PAD | EP4 MK5 80s LRG KTH, CHOR1DEEP, COMP |  | WARMPAD1 PAN1, EQ |  | -0.4 |  | -14.2 |  | 169 |  | R100 |  | 0.1 |  | 0.5 |  | -7.8 |  | +13.5 |  | 200 |  |  | -15.0 |  |  |  |  |  | C6.2 |  |  | N/A |  |  |
| F:34 SOFT CHOIR PAD |  |  | MEN+BOYS OO SOFT FLTR, EQ, DEL, HALLBRT |  |  |  | -8.9 |  | 352 |  | R505 |  |  |  |  |  | -1.5 |  | +0.8 |  | 8.0 kHz |  |  | 0.0 |  |  | D/W2.2, FB1, T108 |  |  |  |  |  | 3.6 |  |  |
| F:35 SOIREE | MOZART FORTEPNO MED KTM, TS, SR, PN, STAGE |  | FLUTE MELLOTRON AMP |  | -2.0 |  | -23.9 |  | 0.0 |  | R75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.5 |  |  |
| F:41 SOUL GRAND | ROYAL GRAND 3D XL KTM, TS, SR, PN, VIBE, EQ, DEL, ROOMBRT |  |  |  | -2.0 |  |  |  |  |  |  |  |  |  | 1.7 |  | -2.8 |  | +3.8 |  | 2.0 kHz |  |  | 0.0 |  |  | D/W4.4, FBOFF, T360 |  |  |  |  |  | 2.4 |  |  |
| F:42 SOFT PLUCK | BLACK UPRIGHT MED KTM, TS, SR, ROOM |  | PICK BASS P 58 FLTR, AMP |  | -1.0 |  | -9.6 |  | 0.0 |  | R33 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8.2 |  |  |
| F:43 PAPER TINES | EP2 IDEAL TIMBRE LRG KTH, ROOM |  | HARP 1 FLTR, AMP, CHOR1 |  | -1.4 |  | -10.9 |  | 0.0 |  | R50 |  |  |  | 0.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5.0 |  |  |
| F:44 SOFT ACCORDION |  |  | VICTORINI ACCORDION 2 FLTR, STAGE |  |  |  | -7.2 |  | 0.0 |  | R33 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.4 |  |  |
| F:45 METALLIC HAMMERS | AMBER UPRIGHT XL KTH, TB, SR, PN, VIBEDEEP, STAGEBRT |  | UKRAINIAN BANDURA FLTR, AMP |  | -3.5 |  | -4.5 |  | 0.0 |  | R144 |  |  |  | 2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4.5 |  |  |
| F:51 SUBTLE VOICE PAD | ROYAL GRAND 3D XL KTH, TB, SR, VIBEDEEP, COMP, HALLBRT |  | SMALL CHOIR FEM AH SOFT PAN1, EQ |  | -1.6 |  | -18.4 |  | 810 |  | R1.94S |  | 0.1 |  | 1.5 |  | 0.0 |  | 0.0 |  | 999 |  |  | -15.0 |  |  |  |  |  | C3.9 |  |  | 4.4 |  |  |
| F:52 FEEL GOOD POP | AMBER UPRIGHT XL KTM, TM, SR, ROOMBRT |  | STUDIOSTRINGS MARC VIB FLTR, AMP, EQ |  | -2.0 |  | -5.4 |  | 0.0 |  | R120 |  |  |  |  |  | 0.0 |  | -9.0 |  | 8.0 kHz |  |  | 0.0 |  |  |  |  |  |  |  |  | 3.4 |  |  |
| F:53 PLAYFUL WURLY | WURLITZER 2 XL KTM, TM, SOFT RELEASE, PAN1, PHAS2DEEP, SMALL, STAGE |  | CELESTE FLTR, EQ |  | -2.5 |  | -8.0 |  | 0.0 |  | R61 |  | 6.4 |  | 2.1 |  | -4.2 |  | -4.8 |  | 482 |  |  | +2.5 |  |  |  |  |  | D0.0 |  |  | 3.3 |  |  |
| PROGRAM |  | PIANO |  | SAMPLE SYNTH |  | PL |  | SSL |  | ATK |  | DC/R |  | FX1 |  | FX2 |  | BASS |  | MID |  |  | FRQ |  |  | TRB |  |  | DELAY |  |  | A/C |  |  | REV |  |  |
| F:54 FLUTE |  |  |  | FLUTE AMP, DEL, HALLBRT |  |  |  | -3.0 |  | 0.8 |  | R33 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | D/W1.4, FBOFF, T96 |  |  |  |  |  | 4.7 |  |  |
| F:55 VISITORS |  | FRENCH HARPSI 1D KTM, SR, CHOR1DEEP, EQ, COMP, STAGEBRT |  | NICELEAD RJp8 AMP, DEL |  | -16.9 |  | -6.0 |  | 0.0 |  | R75 |  |  |  | 0.7 |  | 0.0 |  | -3.2 |  |  | 533 |  |  | 0.0 |  |  | D/W4.0, FB1, T112 |  |  | C10.0 |  |  | 5.2 |  |  |
| G:11 STAGE PIANOS |  | EGRAND 3 MONO SML KTM, EQ, SMALL, STAGE |  | PIANO80 FLTR, AMP, CHOR2DEEP |  | -2.0 |  | -10.1 |  | 0.0 |  | R75 |  |  |  | 0.8 |  | +12.0 |  | -4.5 |  |  | 2.2 kHz |  |  | +2.2 |  |  |  |  |  | D1.5 |  |  | 5.0 |  |  |
| G:12 MELLOW PIZZ |  | MELLOW UPRIGHT MED KTH, TS, SOFT RELEASE, SR, STAGEBRT |  | ORCHSTRINGS PIZZICATO AMP |  | 0.0 |  | -8.0 |  | 0.0 |  | R432 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.3 |  |  |
| G:13 SNOWY EP |  | HYBRID EP SML KTM, DEL, HALLBRT |  | HIMALAYAN FLTR, AMP, CHOR1, EQ |  | -3.0 |  | -15.8 |  | 0.0 |  | R75 |  |  |  | 0.4 |  | 0.0 |  | -5.5 |  |  | 1.8 kHz |  |  | +6.5 |  |  | D/W1.8, FB2, T146 |  |  |  |  |  | 2.0 |  |  |
| G:14 CIMBALOM |  |  |  | CIMBALOM HARD FLTR, AMP, STAGE |  |  |  | -4.2 |  | 0.0 |  | R173 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.6 |  |  |
| G:15 COMBO SOLOIST |  | EP6 SPARKLETOP XL KTM, CHOR2, TWIN, STAGE |  | JAZZ GUITAR FLTR, AMP, EQ |  | 0.0 |  | 0.0 |  | 0.0 |  | R33 |  |  |  | 0.4 |  | -15.0 |  | 0.0 |  |  | 999 |  |  | 0.0 |  |  |  |  |  | D0.0 |  |  | 2.7 |  |  |
| G:21 RESONATING GRAND |  | GRAND IMPERIAL LRG KTL, TS, SR, PN, EQ, HALLBRT |  | CIMBALOM HARD FLTR, AMP |  | 0.0 |  | 0.0 |  | 1.72S |  | R173 |  |  |  |  |  | 0.0 |  | +14.2 |  |  | 8.0 kHz |  |  | 0.0 |  |  |  |  |  |  |  |  | 1.7 |  |  |
| G:22 AMBER STEEL |  | AMBER UPRIGHT XL KTM, SR, PN, STAGEBRT |  | AC GUITAR FINGER FLTR, AMP, EQ |  | -4.0 |  | -6.0 |  | 0.0 |  | R120 |  |  |  |  |  | -6.8 |  | 0.0 |  |  | 2.9 kHz |  |  | +3.5 |  |  |  |  |  |  |  |  | 2.6 |  |  |
| G:23 DYNO 80 |  | EP1 DEEP TIMBRE LRG KTM, TD1, CHOR1DEEP, STAGE |  | PIANO80 FLTR, AMP |  | -4.0 |  | -14.2 |  | 0.0 |  | R75 |  |  |  | 1.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3.5 |  |  |
| G:24 BLOWN SPEAKER |  |  |  | E GUITAR LP 55 FLTR, AMP, VIBEDEEP, EQ, DEL, TWIN, STAGE |  |  |  | -5.0 |  | 8.4 |  | R33 |  |  |  | 2.2 |  | +4.8 |  | -3.8 |  |  | 314 |  |  | -4.2 |  |  | D/W2.9, FB1, T146 |  |  | D7.2 |  |  | 5.1 |  |  |
| G:25 CLOCKWORK |  | ITAL HARPSICH 1D KTM, SR, HALL |  | MBIRA FLTR, AMP, EQ, COMP |  | -5.6 |  | -8.0 |  | 0.0 |  | R173 |  |  |  |  |  | +0.8 |  | -5.5 |  |  | 494 |  |  | +1.8 |  |  |  |  |  | C7.1 |  |  | 2.8 |  |  |
| G:31 MONTUNO |  | ROYAL GRAND 3D XL KTH, TB, SR, PN, COMP, ROOM |  | GRANDMAS UPRIGHT AMP, EQ |  | -2.0 |  | -13.0 |  | 0.0 |  | D2.51S |  |  |  |  |  | -15.0 |  | +5.5 |  |  | 8.0 kHz |  |  | +14.2 |  |  |  |  |  | C1.9 |  |  | 4.1 |  |  |
| G:32 DREAMY UPRIGHT |  | GRAND UPRIGHT LRG KTM, TS, SR, PN, VIBE, EQ, COMP, HALL |  |  |  | 0.0 |  |  |  |  |  |  |  |  |  | 2.5 |  | -7.8 |  | +13.0 |  |  | 8.0 kHz |  |  | -4.0 |  |  |  |  |  | C5.4 |  |  | 3.1 |  |  |

| PROGRAM | PIANO | SAMPLE SYNTH | PL | SSL | ATK | DC/R | FX1 | FX2 | BASS | MID | FRQ | TRB | DELAY | A/C | REV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G:33 FINGERPICKING EP | EP2 IDEAL TIMBRE LRG KTH, TM, PN, DIST, HALLBRT | AC GUITAR FINGER FLTR, AMP | -1.6 | -7.6 | 0.0 | R75 |  |  |  |  |  |  |  | D0.3 | 4.0 |
| G:34 RIFF SYNTH |  | G2 ANACLASSIC 1 FLTR, CHOR2, STAGE |  | -5.0 | 0.0 | R41 |  | 0.6 |  |  |  |  |  |  | 2.8 |
| G:35 UPRIGHT Bs MK I | EP1 DEEP TIMBRE LRG SYNT/PNOF3W6, KTM, TM, PN, PAN1, VIBE, EQ, DIST, STAGE | CONTRA BASS FINGER FLTR | -5.0 | -2.0 | 0.0 | R33 | 3.6 | 2.5 | 0.0 | -2.0 | 447 | 0.0 |  | D1.6 | 2.5 |
| G:41 CHAMBER ENSEMBLE | MOZART FORTEPNO MED KTM, TM, SR, PN, ROOMBRT | FLUTES + OBOE LEGATO AMP | 0.0 | -7.8 | 0.0 | R75 |  |  |  |  |  |  |  |  | 10.0 |
| G:42 UPRIGHT DUO | GRAND UPRIGHT LRG SYNT/PNOF3W6, KTH, TS, SR, PN, EQ, HALL | CONTRA BASS FING VIB FLTR, AMP | 0.0 | -2.0 | 0.0 | R33 |  |  | -8.0 | +15.0 | 8.0 kHz | 0.0 |  |  | 1.7 |
| G:43 SHALLOW DYNO | EP3 SHALLOW CLOSE LRG KTM, TD2, PN, COMP, STAGEBRT |  | 0.0 |  |  |  |  |  |  |  |  |  |  | C5.6 | 1.7 |
| G:44 HORNS AND WINDS |  | HORNS + WINDS LEGATO FLTR, EQ, HALL |  | -5.0 | 0.0 | R120 |  |  | 0.0 | +5.0 | 8.0 kHz | 0.0 |  |  | 2.5 |
| G:45 VELVET VIOLIN | VELVET GRAND LRG PNO/SYNTC5W12, KTM, TB, SR, PN, HALL | SOLO VIOLIN 2 VIBRATO FLTR, AMP | -1.0 | -4.5 | 0.0 | D9.54S |  |  |  |  |  |  |  |  | 3.8 |
| G:51 EGRAND STRINGS | EGRAND 3 MONO SML KTM, TB, CHOR1, STAGEBRT | 11-VIOLAS LEGATO AMP | -2.0 | -20.5 | 1.38S | R1.13S |  | 0.4 |  |  |  |  |  |  | 3.6 |
| G:52 SOFT NAILS | HONKYTONKUPRIGHT MED KTH, TS, SR, PN, EQ, DEL, COMP, HALLBRT |  | 0.0 |  |  |  |  |  | 0.0 | -15.0 | 8.0 kHz | -15.0 | D/W1.3, FB1, T72 | C7.6 | 2.6 |
| G:53 AMPED SPLIT | EP7 AMPED TINES MED SYNT/PNOF3W6, KTM, TB, PN, STAGE | FINGER BASS P 58 FLTR, AMP, EQ, DIST | 0.0 | -7.0 | 2.1 | R33 |  |  | -10.0 | 0.0 | 999 | 0.0 |  | D0.9 | 2.6 |
| G:54 HOLLOWBODY GTR |  | JAZZ GUITAR FLTR, AMP, CHOR1, EQ, DEL, TWIN, HALLBRT |  | -5.0 | 0.0 | R33 |  | 0.7 | +15.0 | +15.0 | 291 | -8.0 | D/W1.9, FB1, T146 | D0.1 | 3.5 |

| PROGRAM |  | PIANO |  | SAMPLE SYNTH |  | PL |  | SSL |  | ATK | DC/R | FX1 | FX2 | BASS | MID | FRQ | TRB | DELAY | A/C | REV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G:55 SUNSHINE LAYERS | BAMBINO UPRIGHT LRG KTM, SR, PN, EQ, DEL, STAGEBRT |  | AC GUITAR 12 STR FLTR, AMP |  | -2.0 |  | -4.0 |  | 0.0 |  | R100 |  |  | -15.0 | 0.0 | 375 | +1.5 | D/W1.2, FB1, T146 |  | 4.5 |  |
| H:11 VELVET FLUTTER | VELVET GRAND LRG KTM, SR, PN, STAGE |  | ALTO RECORDER FLTR, TREM1, CHOR2 |  | -2.0 |  | -21.4 |  | 0.8 |  | R120 | 4.3 | 1.8 |  |  |  |  |  |  | 2.4 |  |
| H:12 SYNTH PANS |  | MARIMBA KTM, SOFT RELEASE, STAGEBRT |  | LITELY NL3 STE FLTR, AMP |  | -2.0 |  | -8.2 |  | 0.0 | R173 |  |  |  |  |  |  |  |  | 3.2 |
| H:13 MK V BITE |  | EP4 MK5 80s LRG KTL, TB, PN, VIBE, EQ, SMALL, STAGE |  |  |  | -3.0 |  |  |  |  |  |  | 1.8 | +5.0 | +3.8 | 2.0 kHz | 0.0 |  | D1.0 | 0.9 |
| H:14 VERY SOFT |  |  |  | SOFT STRINGS LEGATO AMP, EQ, HALLBRT |  |  |  | -6.0 |  | 0.0 | R683 |  |  | 0.0 | 0.0 | 999 | -12.0 |  |  | 5.9 |
| H:15 EP MACHINE PAD |  | EP1 DEEP TIMBRE LRG KTM, TS, HALL |  | EMINENT 310 STR L 8+4 FLTR, AMP, EQ, DEL |  | -3.0 |  | -15.2 |  | 947 | R983 |  |  | -6.2 | +15.0 | 404 | -11.0 | D/W4.8, FB2, T104 |  | 5.0 |
| H:21 BRIGHT 80 |  | BRIGHT GRAND MED KTM, TB, SR, PN, DEL, STAGEBRT |  | PIANO80 FLTR, AMP, EQ |  | -2.5 |  | -8.0 |  | 0.0 | R75 |  |  | -10.0 | +6.8 | 8.0 kHz | 0.0 | D/W1.0, FB1, T138 |  | 3.5 |
| H:22 ORGAN FLUTE |  |  |  | ORGAN FLUTE 3 EQ, HALL |  |  |  | -12.5 |  | 3.5 | R21 |  |  | 0.0 | 0.0 | 2.9 kHz | 0.0 |  |  | 7.0 |
| H:23 PARLOUR GUITAR |  |  |  | PARLOUR GUITAR FLTR, AMP, STAGE |  |  |  | -1.6 |  | 0.0 | R75 |  |  |  |  |  |  |  |  | 3.8 |
| H:24 STRINGS BRASS |  |  |  | STRINGS-BRASS FLTR, AMP, HALLBRT |  |  |  | -7.0 |  | 0.0 | R120 |  |  |  |  |  |  |  |  | 4.0 |
| H:25 FLUTE SECTION |  |  |  | 3-FLUTES LEGATO AMP, STAGE |  |  |  | -4.0 |  | 0.0 | R75 |  |  |  |  |  |  |  |  | 4.6 |
| H:31 BRIGHT ACCORDION |  |  |  | VICTORINI ACCORDION 1 AMP, ROOM |  |  |  | -6.4 |  | 0.0 | R33 |  |  |  |  |  |  |  |  | 5.4 |
| H:32 SMALL CHOIR MALE |  |  |  | SMALL CHOIR MALE OH SOFT AMP, STAGEBRT |  |  |  | -3.0 |  | 0.0 | R120 |  |  |  |  |  |  |  |  | 5.5 |
| H:33 KALIMBA |  |  |  | KALIMBA FLTR, AMP, ROOM |  |  |  | -1.0 |  | 0.0 | R75 |  |  |  |  |  |  |  |  | 7.0 |
| H:34 LADDER BASS |  |  |  | LADDERBASS CHOR2 |  |  |  | -8.0 |  | 0.0 | R21 |  |  |  |  |  |  |  |  | N/A |

| PROGRAM | PIANO | SAMPLE SYNTH | PL | SSL | ATK | DC/R | FX1 | FX2 | BASS | MID | FRQ | TRB | DELAY | A/C | REV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H:35 PUMP ORGAN |  | PUMPORGAN2 LANGEGRD AMP, ROOM |  | -8.2 | 0.0 | R33 |  |  |  |  |  |  |  |  | 6.5 |  |
| H:41 PIZZICATO |  | ORCHSTRINGS PIZZICATO AMP, EQ, HALL |  | -5.0 | 0.0 | R983 |  |  | 0.0 | +2.0 | 8.0 kHz | 0.0 |  |  | 2.6 |  |
| H:42 FRENCH HORNS |  | FRENCH HORNS 1 LEG FLTR, HALL |  | -2.8 | 0.2 | R206 |  |  |  |  |  |  |  |  | 3.5 |
| H:43 METALLOPHONE |  | METALLOPHONE FLTR, AMP, PAN1, CHOR1, DEL, ROOMBRT |  | -2.5 | 0.0 | R173 | 3.0 | 0.6 |  |  |  |  | D/W1.3, FB1, T42 |  | 7.7 |
| H:44 BRIGHT SYNTH PAD |  | SYNTHPAD1 FLTR, AMP, STAGE |  | -6.0 | 0.0 | R432 |  |  |  |  |  |  |  |  | 3.5 |
| H:45 OBOE SECTION |  | 3-OBOES LEGATO FLTR, AMP, STAGE |  | -3.6 | 0.8 | R75 |  |  |  |  |  |  |  |  | 5.5 |
| H:51 SILKWORM |  | SILKWORM JX-3P FLTR, CHOR2, STAGE |  | -12.2 | 0.0 | R75 |  | 0.7 |  |  |  |  |  |  | 2.4 |
| H:52 ORCH STRINGS |  | ORCHSTRINGS LEGATO AMP, HALL |  | -3.0 | 0.0 | R313 |  |  |  |  |  |  |  |  | 3.8 |
| H:53 CHAMBERLINVIOLIN |  | SOLOVIOLIN CHAMBERLIN AMP, ROOM |  | -4.0 | 0.0 | R75 |  |  |  |  |  |  |  |  | 6.9 |
| H:54 SYNTH STRINGS |  | G2 VA STRINGS AMP, STAGEBRT |  | 0.0 | 0.0 | R505 |  |  |  |  |  |  |  |  | 5.0 |
| H:55 FLUTES CLARINETS |  | FLUTES + CLARINETS LEG AMP, STAGEBRT |  | 0.0 | 0.0 | R120 |  |  |  |  |  |  |  |  | 5.0 |